Wania Zanib

23-NTU-CS-1289

BSAI 5th

Embedded IOT

Week 3 – Assignment 1

Code:

//Week 3 - Assisgnment

// Embedded IoT System Fall-2025

// Name: wania zanib                Reg#: 1289

#include <Arduino.h>

//pins

#define LED1_PIN     2

#define LED2_PIN     15

#define SWITCH1_PIN  25

#define SWITCH2_PIN  14

#define DEBOUNCE_MS  50

#define DEBOUNCE_US  (DEBOUNCE_MS * 1000UL)

//Timer Handles

hw_timer_t* debounceTimer1 = nullptr;

hw_timer_t* debounceTimer2 = nullptr;

//Debounce Flags

volatile bool debounceActive1 = false;

volatile bool debounceActive2 = false;

//Timer ISR for Switch 1

void ARDUINO_ISR_ATTR onDebounceTimer1() {

if (digitalRead(SWITCH1_PIN) == LOW) { //still pressed?

digitalWrite(LED1_PIN, !digitalRead(LED1_PIN)); //toggle LED1

}

debounceActive1 = false; //ready for next press

}

//Timer ISR for Switch 2

void ARDUINO_ISR_ATTR onDebounceTimer2() {

if (digitalRead(SWITCH2_PIN) == LOW) { //still pressed?

digitalWrite(LED2_PIN, !digitalRead(LED2_PIN)); //toggle LED2

}

debounceActive2 = false; //ready for next press

}

//External interrupt for Switch 1

void ARDUINO_ISR_ATTR onSwitch1ISR() {

if (!debounceActive1) {

debounceActive1 = true;

timerAlarm(debounceTimer1, DEBOUNCE_US, false, 0);  //start one-shot timer

}

}

//External interrupt for Switch 2

void ARDUINO_ISR_ATTR onSwitch2ISR() {

if (!debounceActive2) {

debounceActive2 = true;

timerAlarm(debounceTimer2, DEBOUNCE_US, false, 0);  //start one-shot timer

}

}

void setup() {

Serial.begin(115200);

//LEDs

pinMode(LED1_PIN, OUTPUT);

pinMode(LED2_PIN, OUTPUT);

digitalWrite(LED1_PIN, LOW);

digitalWrite(LED2_PIN, LOW);

//Configure switches with internal pull-ups

pinMode(SWITCH1_PIN, INPUT_PULLUP);

pinMode(SWITCH2_PIN, INPUT_PULLUP);

//Attached external interrupts (on button press)

attachInterrupt(SWITCH1_PIN, onSwitch1ISR, FALLING);

attachInterrupt(SWITCH2_PIN, onSwitch2ISR, FALLING);

//Created 1 MHz timers

debounceTimer1 = timerBegin(1000000);

debounceTimer2 = timerBegin(1000000);

//Attached debounce ISRs

timerAttachInterrupt(debounceTimer1, &onDebounceTimer1);

timerAttachInterrupt(debounceTimer2, &onDebounceTimer2);

Serial.println("System Ready — Press Buttons to Toggle LEDs");

}

void loop() {

// Nothing needed here; interrupts do all the work.

}

Output:

no input (initial state):

LED 1 turned on:

LED 2 turned on:

Project Link:

https://wokwi.com/projects/444084918831876097